# KWAME NKRUMAH UNIVERSITY OF SCIENCE & TECHNOLOGY, KUMASI

Mid Semester, End of Semester or Supplementary Examination

PCU/ENG/500 – 2024

## MULTIPLE CHOICE QUESTIONS

1. The value of 'α' for a lossless line is:

a)  0

b)  1

c)  Infinity

d)  Data insufficient

2. If propagation constant is 12<60°, then the value of phase constant and attenuation constant is:

a)  α=6, β=10.39

b)  α=61, β=78

c)  α=12, β=20.6

d)  None of the above

3. If a transmission line with inductive reactance of 41.97 Ω and capacitive reactance of 1132.5Ω is operated at 1 GHz, then its phase constant is:

a)  0.0305

b)  0.3

c)  30.5

d)  0.6

4. If the admittance and the impedance of a transmission line are 100 Ω and 50 Ω respectively, then the value of phase constant β is:

a)  0

b)  0.5

c)  2

d)  50

5. For a lossless line, which of the following is true:

a)  γ=α+jβ

b)  γ=α

c)  γ=jβ

d)  γ=α-jβ

6. A microwave generator at 1.2 GHz supplies power to a microwave transmission line having the parameters R=0.8Ω/m, G=0.8 millisiemen/m, L=0.01μH/m and C=0.4PF/m. Propagation constant of the transmission line is:

a)  0.064 +j0.48

b)  0.0654+j0.480

c)  6.54+j0.048

d)  0.64+j0.48

7. In a certain microwave transmission line, the characteristic impedance was found to be 210∠10°Ω and propagation constant 0.2∠78°. What is the impedance Z of the line, if the frequency of operation is 1 GHz:

a)  35.6+j4.28

b)  0.35+j4.97

c)  0.035+j41.97

d)  9.254+j4.6

8. For a transmission line, L=1.8mh/m, C=0.01pF/m, then the phase constant of the line when operated at a frequency of 1 GHz is:

a)  0

b)  1

c)  0.3

d)  4.243

9. Voltage reflection coefficient can be defined as:

a)  Ratio of amplitude of reflected voltage wave to the transmitted voltage wave

b)  Ratio of amplitude of reflected voltage to the incident voltage wave

c)  Ratio of load impedance to the characteristic impedance of the transmission line

d)  None of the mentioned

10. If a transmission line of a characteristics impedance 50 Ω is terminated with a load impedance of 100 Ω, then the reflection coefficient is:

a)  0.3334

b)  0.6667

c)  1.6

d)  1.333

11. If the reflection coefficient for a transmission line is 0.24, then the return loss in dB is:

a)  -12.39dB

b)  15dB

c)  12.39dB

d)  -15.2dB

12. Expression for VSWR in terms of reflection co-efficient is:

a)  (1/1+|r|)/(1+|r|)

b)  (1-|r|)/(1+|r|)

c)  (1/|r|)/(1+|r|)

d)  (1+|r|)/(1-|r|)

13. If the reflection co-efficient for a transmission line is 0.3, then the VSWR is:

a)  0.5384

b)  1.8571

c)  0.4567

d)  3.6732

14. If a transmission line of characteristic impedance 50 Ω is terminated with a load impedance of 150 Ω, then VSWR is:

a)  0.75

b)  0.5

c)  3

d)  1

15. Input impedance of a short circuited transmission line is:

a)  jZtanβl

b)  jZtanβl

c)  jZcotβl

d)  -jZcotβl

16. If a λ/3 transmission line that has a characteristic impedance of 50 Ω is short circuited, then its input impedance is:

a)  -j100Ω

b)  50Ω

c)  86.60Ω

d)  -j86.60Ω

17. The input impedance of a shorted lossless transmission line of length π/4 is:

a)  0

b)  ∞

c)  Z₀

d)  None of the above

18. The key difference between circuit theory and transmission line theory is:

a)  Circuit elements

b)  Voltage

c)  Current

d)  Electrical size

19. Expression for input impedance of an Open circuited transmission line is:

a)  -jZtanβl

b)  jZtanβl

c)  jZcotβl

d)  -jZcotβl

20. For a λ/2 transmission line, if the characteristic impedance of the line is 50 Ω and the line is terminated with a load of 100 Ω, then its input impedance is:

a)  100Ω

b)  50Ω

c)  88.86Ω

d)  75Ω

21. If a λ/3 transmission line is open circuited and has characteristic impedance of 50 Ω then the input impedance is:

a)  j50Ω

b)  50Ω

c)  j28.86Ω

d)  j28.86Ω

22. Expression for a transmission co-efficient of a transmission line is:

a)  2ZL / (ZL + Z₀)

b)  (ZL · Z₀) / (ZL + Z₀)

c)  2Z₀ / (ZL + Z₀)

d)  (ZL + Z₀) / (ZL · Z₀)

23. For a transmission line, if the reflection coefficient is 0.4, then the transmission coefficient is:

a)  0.4

b)  1.4

c)  0.8

d)  2.8

24. If the transmission coefficient of a transmission line is 1.6, then the reflection co-efficient is:

a)  0.8

b)  0.6

c)  0.4

d)  0.3

25. For a transmission line, if the transmission coefficient is 1.4, then the insertion loss in dB is:

a)  -2.922 dB

b)  29 dB

c)  1.46 dB

d)  -29 dB

26. If a device is passive and contains no anisotropic elements, then the device is _________ network:

a)  Reciprocal

b)  Non reciprocal

c)  Lossless

d)  Lossy

Questions 27–29: A lossless transmission line of electrical length l = 0.3λ is terminated with a complex load impedance as shown in Fig. Q 27-29. (Z₀ = 75 Ω, ZL = 10 – j20 Ω)

27. Find the reflection coefficient at the load:

a)  0.621∠215°

b)  0.621∠359°

c)  0.461∠359°

d)  0.461∠215°

28. What is the reflection coefficient at the input to the line:

a)  0.621∠215°

b)  0.621∠359°

c)  0.461∠359°

d)  0.461∠215°

29. Determine the input impedance:

a)  203-β2 Ω

b)  2.05-β0.52 Ω

c)  0.203-β2.1 Ω

d)  20.3-β52 Ω

Questions 30–35: A uniform, lossless 100 Ω line is connected to a load of 50 – j75 Ω. A single short-circuited stub of 100 Ω characteristic impedance is connected in parallel at a distance d from the load.

Possible values of I, II, III, IV:

I.   0.5+j0.75

II.  0.62 +j0.91

III. 1 +j1.3

IV.  1 –j1.3

30. Determine the normalised load admittance:

a)  I only

b)  II only

c)  III and IV

d)  None of the above

31. The point on the VSWR circle where the stub can be positioned corresponds to an impedance/admittance of:

a)  I only

b)  II only

c)  III and IV

d)  None of the above

32. The shortest distance from the load where the stub can be placed is:

a)  0.104λ

b)  0.195λ

c)  0.036λ

d)  0.390λ

33. The length of the stub at this position is:

a)  0.104λ

b)  0.195λ

c)  0.036λ

d)  0.390λ

34. The other distance from the load where the stub can be placed is:

a)  0.104λ

b)  0.195λ

c)  0.036λ

d)  0.390λ

35. The length of the stub at this position (reference to Question 31) is:

a)  0.104λ

b)  0.195λ

c)  0.036λ

d)  0.390λ

36. For a network to be reciprocal, the S matrix of the network must:

a)  Satisfy the unitary condition

b)  Satisfy conjugacy

c)  Equal its transpose

d)  Have all leading diagonal elements to be zero

37. For a network matched at all ports, the S matrix of the network must:

a)  Satisfy the unitary condition

b)  Satisfy conjugacy

c)  Equal its transpose

d)  Have all leading diagonal elements to be zero

38. For co-axial lines and waveguides, _________ is more preferred:

a)  Open circuited stub

b)  Short circuited stub

c)  Slotted section

d)  Co-axial lines cannot be impedance matched

39. For a network to be loss-free, the S-matrix must:

a)  Satisfy the unitary condition

b)  Satisfy conjugacy

c)  Equal its transpose

d)  Have all leading diagonal elements to be zero

Questions 40–44: The S matrix of a 3-port network is given as:

[S] = (1/√2) | 0   S₁₂  S₁₃ |

| S₂₁  0    1  |

| S₃₁  1    0  |

Noting: I. Unitary   II. Reciprocal   III. Lossless   IV. Matched at all ports

40. The network is:

a)  I and IV only

b)  I and II only

c)  III only

d)  None of the above

41. The network is:

a)  Not physically realizable

b)  A circulator

c)  A Directional coupler

d)  None of the above

42. Setting S₁₂ = S₂₁ will result in a:

a)  A power divider

b)  A circulator

c)  A Directional coupler

d)  None of the above

43. In addition to the condition set in Question 38, if S₁₂ = S₂₁ = S₃₁ = 0, then power incident at port 2 will exit at Port:

a)  3

b)  2

c)  1

d)  Not enough data to determine

44. In addition to the condition set in Question 38, if |S₁₂| = |S₂₁| = |S₃₁| = 1, then power incident at port 2 will exit at Port:

a)  3

b)  2

c)  1

d)  4

45. The major advantage of single stub tuning over other impedance matching techniques is:

a)  Lumped elements are avoided

b)  It can be fabricated as a part of transmission line media

c)  It involves two adjustable parameters

d)  All of the above

46. The two adjustable parameters in single stub matching are distance 'd' from the load to the stub position, and:

a)  Susceptance provided by the stub

b)  Distance of the stub from the load

c)  Distance of the stub from the generator

d)  Reactance provided by the stub

Questions 47–51: Given that a load 200+j100 Ω is to be matched to a line whose characteristics impedance is 100 Ω using an L-section matching network.

47. The load admittance in Siemens is:

a)  0.4+j0.5

b)  0.4+j0.2

c)  0.4+j0.5

d)  0.4+j0.2

48. Moving clockwise from the load admittance on a constant conductance circle will intersect the rotated 1+jx circle at:

a)  0.4+j0.5

b)  1+j1.2

c)  0.4+j0.5

d)  0.4+j0.2

49. Moving counterclockwise from the load admittance on a constant conductance circle will intersect the rotated 1+jx circle at:

a)  0.4+j0.5

b)  1+j1.2

c)  0.4+j0.5

d)  0.4+j0.2

50. What value of susceptance in Siemens in the case of Q48 that is required to reach the rotated 1+jx circle?

a)  -0.7

b)  1.2

c)  -1.2

d)  0.3

51. What value of susceptance in Siemens in the case of Q49 that is required to reach the rotated 1+jx circle?

a)  -0.7

b)  1.2

c)  -1.2

d)  0.3

52. The respective susceptance and reactance that will match the load to the line are:

a)  0.7 and -1.2

b)  -0.7 and 1.2

c)  0.3 and 1.2

d)  -0.3 and -1.2

53. The device represents:

a)  Wilkinson power divider

b)  Isolator

c)  Quadrature hybrid

d)  Rat-race hybrid

54. Compute the directivity in dB:

a)  0

b)  ∞

c)  3

d)  1

55. Compute the Coupling in dB:

a)  0

b)  ∞

c)  3

d)  1

56. Compute the isolation in dB:

a)  0

b)  ∞

c)  3

d)  1

57. If 10 watt is applied to the input port of a standard Wilkinson divider, then the sum of the power measured at the two output ports of the Wilkinson coupler is:

a)  5 watt

b)  10 watt

c)  7.07 watt

d)  8 watt

58. The analysis of Wilkinson coupler is done using:

a)  Even-odd mode analysis

b)  Symmetry

c)  S matrix approach

d)  None of the mentioned

59. For an equal-split Wilkinson power divider of 50Ω system impedance, the characteristic impedance of quarter wave transmission line used is:

a)  70.7 Ω

b)  50 Ω

c)  100 Ω

d)  None of the mentioned

60. In 180° hybrid, different power levels can be received at the two output ports of the hybrid:

a)  True

b)  False

Examiner: A.-R. Ahmed